Liquid heater specification(8kw)

2020

CATALOG
1. Product introduction                         
1.1 Product overview                           
1.1.1 Function                                      
1.1.2 Features                                     
1.2 Product appearance                      
1.3 Product size 
1.4 Product connector and interface definition 
1.5 Product exploded view 
2. Basic electrical performance of the product 
2.1 Basic electrical parameters 
2.2 Temperature 
2.3 Low pressure 
2.4 High pressure 
2.5 High voltage safety 
3. Product control plan 
3.1 Air conditioning control framework 
3.2 Block diagram of control principle 
3.3 Control method 
3.4 Control function 
3.4.1 Soft start function: PWM soft start 
3.4.2 Restricted functions: 
3.4.3 Protection function: 
4. Precautions for product installation 
5. Precautions for the use of PTC water heating heater 
6. Common fault diagnosis, inspection and maintenance of PTC plumbing heater

1. Product introduction
1.1 Product overview
1.1.1 Function
PTC water heating heater can provide heat for the new energy vehicle cockpit and meet the standards of safe defrosting and defogging. At the same time, it provides heat to other vehicles that require temperature adjustment (such as batteries).
1.1.2 Features
Electricity is used to heat the antifreeze, and the heater is used to heat the interior of the car. Installed in the water cooling circulation system
Warm air and temperature controllable
Use PWM to adjust the drive IGBT to adjust the power with short-term heat storage function
Whole vehicle cycle, supporting battery thermal management and environmental protection

1.1.2 Product appearance

1.1.3 Product size

1.1.4 Product connector and interface definition

1.Upper base

7.High voltage

connector

2.Controller

panel

3.Heating core

assembly

4. Water tank

5.Gasket

6.Lower base

8.Low voltage connector

9.Nozzle seal

1.1.5 Product exploded view

According to the voltage requirement of 600V, the PTC sheet is 3.5mm thick and TC210 ℃, which ensures good withstand voltage and durability. The internal heating core of the product is divided into four groups, which are controlled by four IGBTs.       In order to ensure the protection grade of the product IP67, insert the heating core assembly into the lower base obliquely, cover the (Serial No. 9) nozzle sealing ring, and then press the outer part with the pressing plate, and then put it on the lower base     (No. 6) is sealed with pouring glue and sealed to the upper surface of D-type pipe. After assembling other parts, the sealing gasket (No. 5) is used between the upper and lower bases to ensure the good waterproof performance of the product.

2. Basic electrical performance of the product
2.1 Basic electrical parameters

| Description | Condition | Minimum | typical | maximum | unit |
| --- | --- | --- | --- | --- | --- |
| power | a) Test voltage: control voltage: DC24V; load voltage: 600VDC b) ambient temperature: 20℃±2℃; water inlet temperature: 0℃±2℃; flow rate: 10L/min c) Air pressure: 70kPa~106ka No coolant, no connecting wire |  | 8000 |  | W |
| weight | No coolant, no connecting wire |  | 2.7 |  | KG |
| Antifreeze volume |  |  | 170 |  | mL |

  - Temperature

| Description | Condition | Minimum | typical | maximum | unit |
| --- | --- | --- | --- | --- | --- |
| Storage temperature |  | -40 |  | 105 | ℃ |
| Working temperature |  | -40 |  | 105 | ℃ |
| environment humidity |  | 5% |  | 95% | RH |

  - Low pressure

| Description | Condition | Minimum | typical | maximum | unit |
| --- | --- | --- | --- | --- | --- |
| Control voltage VCC |  | 18 | 24 | 32 | V |
| Ground |  |  | 0 |  | V |
| Supply current | Steady state current | 90 | 120 | 160 | mA |
| Starting current |  |  |  | 1 | A |

  - High pressure

| Description | Condition | Minimum | typical | maximum | unit |
| --- | --- | --- | --- | --- | --- |
| Supply voltage | Turn on heating | 480 | 600 | 720 | V |
| Supply current | Nominal condition |  | 13.3 |  | A |
| Inrush current | Nominal condition |  |  | 17.3 | A |
| stand-by current | Nominal condition |  |  | 1.6 | mA |

  - High voltage safety

| 1 | High voltage warning label | have |
| --- | --- | --- |
| 2 | Flame-retardant plastic insulation shell protection to prevent access to high-voltage live parts | have |
| 3 | Basic insulation of high voltage live parts | have |
| 4 | Provide potential equalization grounding point | have |
| 5 | Insulation resistance>50MÙ | have |
| 6 | Passive discharge | have |
| 7 | High voltage interlock | have |
| 8 | High voltage terminals comply with IPXXB protection | have |
| 9 | Meets IP67 protection | have |
| 10 | Comply with 2U+1000V withstand voltage test | have |
| 11 | High pressure and overpressure turn off heating | have |
| 12 | PTC over temperature protection | have |
| 13 | PCB over temperature protection | have |
| 14 | IGBT over temperature protection | have |
| 15 | PTC overcurrent protection | have |
| 16 | PTC short circuit protection | have |
| 17 | IGBT breakdown feedback | have |
| 18 | PTC temperature sensor failure detection, no heating when failure | have |
| 19 | IGBT temperature sensor failure detection, no heating when failure | have |
| 20 | PCB temperature sensor failure detection, no heating when failure | have |
| 21 | Even if the IGBT control fails, the heater temperature does not exceed 200+℃ due to PTC's own characteristics | have |
| 22 | Low voltage anti-reverse connection | have |
| 23 | High voltage reverse connection feedback (reverse connection will cause uncontrolled heating and cause PTC damage) | have |
| 24 | Controller power on and self-check, no heating if there is a fault | have |
| 25 | Communication is interrupted and heating is stopped | have |
| 26 | High voltage acquisition real-time feedback | have |

- Product control scheme

  - Air conditioning control framework

Plumbing PTC

water circulatory system

①The command input is completed by the air conditioner panel.

②The air conditioner panel sends the user's operation commands to the controller through CAN communication or ON/OFF PWM.
③After receiving the command signal, the plumbing PTC controller turns on the PTC in PWM mode according to the power requirements.
Design advantages:

①It adopts 4-channel PWM control mode, the bus impulse current is small, and the requirement for the relay in the whole vehicle loop is low.

②PWM mode control enables continuous power adjustment.

③The CAN communication mode can report the working status of the controller, which is convenient for the control and monitoring of the whole vehicle.

  - Block diagram of control principle

3.4 Control mode CAN communication
3.5 Control function
3.5.1 Soft start function: PWM soft start
PWM uses 30 seconds from 0% to 100%. This function is used to protect the high inrush current and sudden changes on the high voltage bus. Each channel is gradually soft-started, and there is ripple current during the soft-start process, and the inrush current is small (<1.3 times of the rated current).
3.5.2 Restricted function:
3.5.2.1 Temperature limit

①When the temperature of the water outlet reaches 80℃, the power will be gradually reduced to keep the temperature of the water outlet relatively constant
② When the temperature of any one of the IGBTs reaches 85°C, it will take the initiative to gradually reduce the power or shut down, and resume operation when it is lower than 80°C so that the IGBT temperature does not exceed the limit
3.3.1.1 Power limit

①Soft start power limit: The power of each channel is limited to 3800W during soft start

3.3.2 Protection function:

3.3.2.1 High-voltage over-voltage protection

The operating voltage range can be set according to customer requirements. The heaters that are not within the range will not start or shut down, and will be soft-started to the current set value after returning to the range.
3.3.2.2 IGBT drive voltage overvoltage protection

When the IGBT drive voltage is not within the set working range, the heating will be all shut down, and the soft start will be restarted to the current set value after recovery.
3.3.2.3 Low voltage reverse connection protection Low voltage reverse connection does not work.
3.3.2.4 Outlet water temperature over-temperature protection
When the water outlet temperature exceeds 105℃ for 3 seconds, all heating will be turned off immediately
3.3.2.5 Short circuit protection
If any PTC circuit is short-circuited, the circuit will be closed immediately, and the test will be repeated 5 times, but the circuit is still short-circuited, and the circuit will not be heated until it is powered on again.
3.3.2.6 Overcurrent protection
If the circuit exceeds 2 times the rated current for 5s during heating operation, it is considered as overloaded and the circuit is
Will be shut down and restarted.

Warning: High voltage does not have the ability to prevent reverse connection. High voltage reverse connection will cause direct heating to be uncontrolled. High voltage reverse connection may cause PTC damage, may cause the vehicle to fail to be precharged and fail to start, and may cause the precharge resistor to burn!

- Precautions for product installation

4.2 Before turning on the power, pay attention to the installation and the reliable connection of the connectors (the main high voltage cannot be reversed, otherwise the control will fail, and the control board will be damaged for more than 1 minute)
4.3 The ground wire is connected reliably to prevent static electricity from damaging the components
4.4 Antifreeze should not contain impurities to prevent blocking the water tank
4.5 After unpacking, please be sure to check whether there is any appearance damage caused by transportation. Damage caused by improper installation and use (including use and installation conditions outside the scope of the specification) is not covered by the warranty.
4.6 Product installation direction: The heater can be installed in other directions except that the water inlet and outlet cannot be downward at the same time.

5 Precautions for the use of PTC plumbing heater

5.2 After the heater is connected correctly, it is recommended to turn on low-voltage power and then high-voltage when powering on; when power-off, turn off high-voltage power and then low-voltage power.
5.3 The water flow rate in the water cycle is ≥4L/min. If the flow rate is too low, it will cause frequent temperature protection.

6 Common fault diagnosis, inspection and maintenance of PTC plumbing heater

| Failure phenomenon | Cause of failure | Corrective measures |
| --- | --- | --- |
| Can not be used | The high-voltage plug-in is not inserted or has poor contact | Check whether the high-voltage connector is in good contact |
|  | Low-voltage plug-in is not inserted or has poor contact | Check whether the low voltage connector is in good contact |
|  | Insulation resistance decreased | Replace PTC plumbing heater |
| No heating | High-voltage plug-in is damaged or poor contact | Check whether the high-voltage connector is in good contact |
|  | Low-voltage plug-in is damaged or poor contact | Check whether the low voltage connector is in good contact |
|  | Low voltage plug-in wiring sequence error | Check the wiring sequence of the low voltage connector |
|  | The PWM signal is not within the scope of the agreement | Adjust the PWM duty cycle within the adjustment range |
|  | PWM signal wire is shorted to ground | Check whether the signal terminal pins of the low-voltage connector are skewed and short-circuited, |
|  | PWM signal wire is shorted to power supply | Check whether the signal terminal pin of the low-voltage connector is skewed and short-circuited |
|  | Low pressure is not in range | Check voltage |
|  | High pressure is not in range | Check voltage |

|  | Internal sensor failure | Replace PTC plumbing heater |
| --- | --- | --- |
|  | Internal control circuit failure | Replace PTC plumbing heater |
| Slow heating | Low water temperature slow start | no |
|  | Low PWM power request | Adjust the PWM duty cycle |
|  | Low high voltage voltage slow start | Adjust voltage |
|  | Low antifreeze flow | Adjust flow |
|  | Waterway is blocked | Cleaning the waterway |
|  | Partial failure of internal control circuit | Replace PTC plumbing heater |
| Direct heating without control signal | High voltage reverse connection | High voltage polarity can be changed within 1 minute |
|  | Internal control circuit failure | Replace PTC plumbing heater |
| Automatic shutdown and restart | Overcurrent | no |
|  | Over temperature | Reduce request power or increase traffic |
|  | PWM is sometimes out of range | Adjust the PWM duty cycle |
|  | Low pressure overpressure | Adjust voltage |
|  | Low pressure | Adjust voltage |
|  | High pressure overpressure | Adjust voltage |
|  | High pressure and undervoltage | Adjust voltage |
|  | Internal control circuit failure | Replace PTC liquid heater |
| External fuse blown | The heating core is short-circuited and the control circuit protection fails | Replace PTC liquid heater |
|  | The high voltage positive and negative poles are short-circuited to the shell at the same t | Replace PTC liquid heater |